Submitted By:

Umair Umar CE-25-107

Assignment:

CA Lab No. 10

Submitted To:

Mr. Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

Circuit Analysis Lab 10

Verification of Thevenin’s Theorem

Step-by-Step Proteus ISIS Simulation Procedure

Overview

This document walks you through verifying Thevenin’s Theorem in Proteus ISIS. You will build three separate circuits in sequence, measure key values from each, and confirm that the Thevenin equivalent circuit produces the same load current as the original network.

The circuit uses three 330Ω resistors and a 1kΩ load. With a 5V supply you should obtain:

- VTH = 2.5 V

- RTH = 495.05 Ω

- (same from both original and equivalent circuit) IL = 1.67 mA

Part 1 — Build the Original Circuit and Measure IL

Step 1: Open Proteus and Create a New Project

- Launch Proteus ISIS.

- Go to File → New Project. Name it “Thevenin_Lab10” and click Finish.

- You will land on a blank schematic sheet.

Step 2: Add Components

- Press the P key (or click the component picker icon) to open the component library.

- Search for and place the following components onto the schematic:

| Component | Search Term | Notes |
| --- | --- | --- |
| DC Voltage Source | VSOURCE | Set to 5V |
| Resistor (x3 for 330Ω) | RES | Set value to 330 |
| Resistor (1kΩ load) | RES | Set value to 1k |
| DC Ammeter | DC AMMETER | From Virtual Instruments |
| DC Voltmeter | DC VOLTMETER | From Virtual Instruments |

| ! | To find Virtual Instruments (ammeter/voltmeter): click the “Virtual Instruments” icon on the left toolbar (it looks like an oscilloscope). DC AMMETER and DC VOLTMETER are in that list. |
| --- | --- |

Step 3: Wire the Original Circuit

Connect the components as follows (refer to the circuit diagram in your lab sheet):

- Place the 5V source on the left. Connect its positive terminal to the top-left wire.

- Connect a 330Ω resistor in series along the top wire from the source positive terminal to a junction node (node A-internal).

- From that junction, connect another 330Ω resistor going downward to the bottom (ground) wire. This is the middle shunt resistor.

- From the same junction, connect the third 330Ω resistor along the top wire to the right, reaching terminal A.

- Connect the DC Ammeter in series after the right 330Ω resistor (between it and the 1kΩ load).

- Connect the 1kΩ load resistor from the ammeter’s other terminal down to the bottom (ground) wire. This completes the loop.

- Connect the negative terminal of the source to the bottom ground wire.

| i | Wire connections: hover over a component pin until a green dot appears, then click and drag to the destination pin. A green line confirms a valid connection. |
| --- | --- |

Step 4: Set Component Values

- Double-click the voltage source. Set its voltage to 5V. Click OK.

- Double-click each 330Ω resistor and confirm the resistance value is 330 (Proteus assumes ohms by default).

- Double-click the 1kΩ resistor and set it to 1k.

Step 5: Run the Simulation and Record IL

- Press the Play button (green triangle at the bottom left) to start the simulation.

- Read the ammeter display. You should see approximately 1.67 mA.

- Record this as IL (Circuit-I) in your lab table.

| IL (Circuit - I) | 1.67 mA |
| --- | --- |

| ! | If the ammeter reads a negative value, it means current is entering its negative terminal. Simply double-click the ammeter and swap its terminals, or rotate it 180°. |
| --- | --- |

Part 2 — Measure VTH (Open Circuit Voltage)

Step 6: Remove the Load Resistor

- Stop the simulation (press the Stop button).

- Delete the 1kΩ load resistor from the schematic. Terminals A and B are now open-circuited.

- Also remove the ammeter (it serves no purpose here since no current flows through the open terminal).

| i | The remaining circuit is: 5V source → top-left 330Ω → junction → middle 330Ω to ground, and top-right 330Ω going to open terminal A. |
| --- | --- |

Step 7: Place the Voltmeter Across Terminals A–B

- Place the DC Voltmeter across the open terminals A and B (positive probe to A, negative probe to B/ground).

Step 8: Run and Record VTH

- Press Play to run the simulation.

- Read the voltmeter. You should see 2.5 V.

- Record this as VTH in your lab table.

| VTH | 2.5 V |
| --- | --- |

| ! | VTH = 2.5V because the open terminal means no current flows through the right 330Ω, so no voltage drop across it. The voltmeter reads the voltage across the middle 330Ω, which is a voltage divider between the top-left 330Ω and the middle 330Ω: VTH = 5V × 330/(330+330) = 2.5V. |
| --- | --- |

Part 3 — Measure RTH (Thevenin Resistance)

Step 9: Modify the Circuit to Find RTH

RTH is found by deactivating (shorting) the voltage source and applying a test voltage across the open terminals A–B.

- Stop the simulation.

- Remove the voltmeter.

- Replace the 5V source with a plain wire (short circuit). To do this: delete the source and draw a wire directly connecting its two terminals (top and bottom rails on the left).

- Connect a new 5V DC source between terminals A and B, with the positive terminal at A.

- Place the DC Ammeter in series between the positive terminal of this test source and terminal A.

| i | The circuit the test source now “sees” looking in through A–B is: right 330Ω in series with (middle 330Ω ∥ left 330Ω). This is exactly RTH. |
| --- | --- |

Step 10: Run and Record RTH

- Press Play to run the simulation.

- Read the ammeter. You should see approximately 10.1 mA.

- Calculate: RTH = Vtest / I = 5 / 0.0101 ≈ 495.05 Ω

- Record this in your lab table.

| RTH | 5V ÷ 10.1mA ≈ 495.05 Ω |
| --- | --- |

| ! | If the ammeter reads −10.1 mA (negative), just flip the ammeter. The magnitude is correct — it is only a polarity issue caused by the ammeter orientation. |
| --- | --- |

Part 4 — Build the Thevenin Equivalent Circuit

Step 11: Clear the Schematic

- Stop the simulation.

- Select all (Ctrl+A) and delete, or start a new sheet. You will now draw a much simpler circuit.

Step 12: Build the Equivalent Circuit

The Thevenin equivalent circuit is simply VTH in series with RTH driving the load:

- Place a DC voltage source and set it to 2.5V (your measured VTH).

- Connect a resistor in series and set it to 495 Ω (your measured RTH). You can use a potentiometer/DRB set to 495Ω, or simply a resistor.

- Connect the DC Ammeter in series after the RTH resistor.

- Connect the 1kΩ load resistor from the ammeter’s other terminal to ground.

- Connect the negative terminal of the source to ground to complete the loop.

| i | The circuit reads: VTH (2.5V) → RTH (495Ω) → Ammeter → RL (1kΩ) → back to source negative. |
| --- | --- |

Step 13: Run and Record IL from Equivalent Circuit

- Press Play.

- Read the ammeter. You should see approximately 1.67 mA — matching the original circuit exactly.

- Record this as IL (Equivalent Circuit) in your lab table.

| IL (Equivalent Circuit) | 1.67 mA |
| --- | --- |

| ! | If you get double the expected current (e.g. 3.34 mA), check that your source is set to VTH = 2.5V and not the original 5V. This is a common mistake when copying the circuit. |
| --- | --- |

Verification

Thevenin’s Theorem is verified when the load current from the equivalent circuit matches the load current from the original circuit. Check your results against the theoretical formula:

IL = VTH / (RTH + RL) = 2.5 / (495.05 + 1000) = 2.5 / 1495.05 ≈ 1.67 mA

| Parameter | E (V) | VTH (V) | RTH (Ω) | IL Orig. (mA) | IL Equiv. (mA) |
| --- | --- | --- | --- | --- | --- |
| Theoretical | 5 | 2.5 | 495 | 1.67 | 1.67 |
| Measured (Proteus) | 5 | 2.5 | 495.05 | 1.67 | 1.67 |

Since IL from the original circuit equals IL from the Thevenin equivalent circuit, the theorem is verified.

Common Mistakes and Fixes

| Problem | Fix |
| --- | --- |
| Ammeter reads negative | Flip/rotate the ammeter 180°. Magnitude is still correct. |
| Ammeter reads 0 | Ammeter is not in series — check wiring. It must be in the current path, not across components. |
| Equivalent circuit IL is double (e.g. 3.34 mA) | The source in the equivalent circuit is still set to 5V instead of VTH = 2.5V. Update it. |
| RTH measurement gives wrong value | Check that the original source is fully replaced by a wire (shorted), not just deleted leaving an open. |
| VTH reads 0V | The 1kΩ load was not removed. VTH must be measured with the load disconnected (open circuit). |
| Simulation won’t start / errors | Every component must have both terminals connected. Dangling wires cause simulation errors. Check all pins. |