Submitted By:

Umair Umar CE-25-107

Assignment:

CA Lab No. 10

Submitted To:

Mr. Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

Circuit Analysis (Lab10)

Experiment 9: Verification of Thevenin’s Theorem

|  |  | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) | Report Marks (3) Total marks (10) |
| --- | --- | --- | --- | --- | --- | --- |
| CLO2 | Use with identification the circuit components, breadboards, multimeter, power supplies, signal generators, and oscilloscopes. |  |  |  |  |  |
| CLO3 | Make measurements in their own constructed electric circuits. |  |  |  |  |  |

Objective

The objective of this lab is to verify Thevenin’s theorem and to find the full load current.

Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1k, 330 ohms | 1, 3 |
| 7 | Potentiometer | Standard | 1 |

Description

Statement:

Any linear bilateral, active two terminal network can be replaced by an equivalent voltage source (VTH), Thevenin’s voltage or VOC in series with looking back resistance RTH.

Precautions

1. Voltage control knob of RPS should be kept at minimum position.

2. Current control knob of RPS should be kept at maximum position.

Procedure

1. Connections are given as per the circuit diagram.

2. Set a particular value of voltage using RPS and note down the corresponding ammeter readings.

3. To find VTH: Remove the load resistance and measure the open circuit voltage using multimeter (VTH).

4. To find RTH: Remove the RPS and short circuit it and find the RTH using multimeter.

5. Give the connections for equivalent circuit and set VTH and RTH and note the corresponding ammeter reading.

6. Verify Thevenin’s theorem.

Circuit - 1: To find load current

[Circuit diagrams as shown in original lab sheet: original circuit, VTH measurement circuit, RTH measurement circuit, and Thevenin equivalent circuit]

Thevenin Practical Values

|  | E(V) | VTH(V) | RTH (ohms) | IL (mA) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Circuit - I | Equivalent Circuit |
| Values | 5 V | 2.5 V | 495.05 Ω | 1.67 mA | 1.67 mA |

Post Lab Questions

1.  What is Thevenin’s Theorem?

Thevenin’s Theorem states that any linear, bilateral, active two-terminal network can be replaced by an equivalent circuit consisting of a single voltage source (VTH) in series with a resistance (RTH). VTH is the open-circuit voltage across the two terminals, and RTH is the equivalent resistance seen from the terminals with all independent sources deactivated (voltage sources shorted, current sources open-circuited). The load current is then given by:

IL = VTH / (RTH + RL)

2.  What are the applications of Thevenin’s Theorem?

1. Circuit simplification: Complex networks with a variable load can be reduced to a single VTH and RTH, making load analysis much easier without re-solving the entire network each time the load changes.

2. Maximum power transfer: RTH is used to determine the condition for maximum power delivery to a load; maximum power is transferred when RL = RTH.

3. Electronic circuit design: Used in transistor biasing, amplifier analysis, and sensor interface circuits to predict how a component will behave when connected to a larger network.

4. Power systems: Thevenin equivalents are used to model the behavior of transmission lines and generators as seen by a connected load.

3.  What are the theoretical and practical limitations of Thevenin’s Theorem?

Theoretical Limitations:

1. Applicable only to linear bilateral networks; nonlinear elements such as diodes and transistors invalidate the theorem directly.

2. Cannot be directly applied to circuits with dependent sources unless they are handled carefully using test-source analysis.

3. Assumes DC or single-frequency AC steady-state conditions; the theorem is not directly valid for transient or multi-frequency signals.

Practical Limitations:

1. Component tolerances cause the measured VTH and RTH to differ from their theoretical values, introducing errors in the predicted load current.

2. Measuring RTH using the test-source method can be affected by the internal resistance of the measuring instruments (ammeter, voltmeter), introducing additional error.

3. In simulation environments like Proteus, ideal components give exact results, but in real hardware, stray capacitance and inductance introduce errors, especially at higher frequencies.

4. The theorem assumes a two-terminal linear network; circuits with multiple coupled ports or feedback paths require modified approaches.