Submitted By:

Umair Umar CE-25-107

Assignment:

CA Lab No. 7

Submitted To:

Mr. Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

Circuit Analysis - Lab 7

Proteus ISIS Simulation Procedure

Inverting and Non-Inverting Operational Amplifier Configurations

Section 1 — Setting Up the Proteus Workspace

- Open Proteus ISIS. Go to File > New Project and create a new schematic design.

- Click the Component Mode button (the small IC chip icon on the left toolbar) or press P to open the Pick Devices dialog box.

- Search for and add the following components to your library by typing each name in the search bar and double-clicking to select it:

| Component Name | Proteus Search Term | Purpose |
| --- | --- | --- |
| VSINE | VSINE | Sinusoidal voltage source (signal generator) |
| Resistor | RES | Input and feedback resistors (R1, R2) |
| Op-Amp (741) | UA741 | IC 741 operational amplifier |
| Oscilloscope | OSCILLOSCOPE | To display input and output waveforms |

- Also add power terminals: go to Terminals Mode (the flag icon on the left toolbar). Select POWER and place two terminals on the schematic — one for +VCC and one for −VCC. Select GROUND and place a ground terminal as well.

- Close the Pick Devices dialog. All selected components will appear in the Devices list on the left panel, ready to be placed.

Section 2 — Configuring the VSINE Source

The VSINE source acts as the function generator. Both tasks use the same input signal settings.

- Place the VSINE component on the schematic by clicking it from the left panel and clicking on the workspace to place it.

- Double-click the VSINE component to open its properties.

- Set the following parameters:

| Amplitude (VAMPL) | 1 V (this gives 2 V peak-to-peak) |
| --- | --- |
| Frequency (FREQ) | 40 Hz |
| DC Offset (VOFF) | 0 V |
| Phase (PHASE) | 0 |
| AC Amplitude | Leave blank or 0 |

Note: Amplitude in VSINE means peak voltage, not peak-to-peak. Setting 1 V amplitude gives a 2 V p-p waveform, which matches the lab requirement.

- Click OK to confirm.

Section 3 — Task 1: Inverting Amplifier

Gain = −R2 / R1 = −3 kΩ / 1 kΩ = −3    |    Expected output: 6 V p-p, inverted (180° phase shift)

Wiring the Circuit

- Place the UA741 op-amp in the center of the schematic.

- Place R1 (set value to 1k) to the left of the op-amp. Connect one end of R1 to the VSINE output (positive terminal of the source). Connect the other end of R1 to pin 2 (the inverting terminal, marked −) of the UA741.

- Place R2 (set value to 3k) as the feedback resistor. Connect one end to pin 2 (inverting terminal) and the other end to pin 6 (output) of the UA741. This is the feedback path.

- Connect pin 3 (non-inverting terminal, marked +) of the UA741 directly to the GROUND terminal.

- Connect pin 7 of the UA741 to the +VCC power terminal. Double-click the +VCC terminal and set its voltage to +12V.

- Connect pin 4 of the UA741 to the −VCC power terminal. Double-click the −VCC terminal and set its voltage to −12V.

- Connect the negative terminal of the VSINE source to the GROUND terminal.

Note: Pin 1, pin 5, and pin 8 of the UA741 are left unconnected. This is correct — they are offset null and no-connect pins.

Connecting the Oscilloscope

- Place the OSCILLOSCOPE on the schematic (from the Virtual Instruments menu or the component list).

- Connect Channel A (input A) of the oscilloscope to the junction between VSINE and R1. This will display the input signal.

- Connect Channel B (input B) of the oscilloscope to pin 6 (output) of the UA741. This will display the output signal.

- Connect the ground of the oscilloscope to the circuit ground terminal.

Running the Simulation

- Press the Play button (green triangle at the bottom left) to start the simulation.

- The oscilloscope window will open. You should see two waveforms: Channel A showing the input sine wave and Channel B showing the inverted and amplified output.

- Adjust the Time/Div knob on the oscilloscope. Since the frequency is 40 Hz, one period = 25 ms. Set Time/Div to 5 ms/div to display approximately 2 full cycles.

- Adjust the Volts/Div for Channel A to 0.5 V/div or 1 V/div (input is 2 V p-p). Adjust Volts/Div for Channel B to 2 V/div (output is 6 V p-p).

- Verify: the output waveform should be flipped upside down (180° phase shift) and have 3 times the amplitude of the input.

- To verify gain: measure Vout p-p and Vin p-p from the oscilloscope. Calculate |Av| = Vout / Vin. Result should be 3.

Section 4 — Task 2: Non-Inverting Amplifier

Gain = 1 + R2 / R1 = 1 + 2 kΩ / 1 kΩ = +3    |    Expected output: 6 V p-p, same phase as input

Note: You can either build this as a new schematic or modify the Task 1 circuit. The key differences are: the input connection point, the feedback resistor value, and R1 goes to ground instead of the input.

Wiring the Circuit

- Place the UA741 op-amp in the center of the schematic.

- Connect the positive terminal of VSINE directly to pin 3 (non-inverting terminal, marked +) of the UA741. This is the key difference from the inverting circuit — the input goes to the + terminal, not through R1 to the − terminal.

- Place R1 (set value to 1k). Connect one end to pin 2 (inverting terminal) of the UA741 and the other end to GROUND.

- Place R2 (set value to 2k) as the feedback resistor. Connect one end to pin 2 (inverting terminal) and the other end to pin 6 (output) of the UA741.

- Connect pin 7 to +VCC (+12V) and pin 4 to −VCC (−12V), same as Task 1.

- Connect the negative terminal of VSINE to GROUND.

Connecting the Oscilloscope

- Connect Channel A of the oscilloscope to the junction between VSINE and pin 3 of the UA741 to display the input.

- Connect Channel B of the oscilloscope to pin 6 (output) of the UA741 to display the output.

Running the Simulation

- Press Play to start the simulation.

- On the oscilloscope, set Time/Div to 5 ms/div and adjust Volts/Div as before.

- Verify: the output waveform should be in phase with the input (both peaks at the same time) and have 3 times the amplitude.

- Confirm gain: measure Vout p-p and Vin p-p. Calculate Av = Vout / Vin = 3.

Section 5 — Key Differences Summary

|  | Inverting Amplifier | Non-Inverting Amplifier |
| --- | --- | --- |
| Input connection | VSINE → R1 → pin 2 (−) | VSINE → pin 3 (+) directly |
| R1 connects | VSINE to pin 2 | Pin 2 to ground |
| R2 (feedback) | Pin 2 to pin 6 (3 kΩ) | Pin 2 to pin 6 (2 kΩ) |
| Pin 3 connection | To ground | To VSINE input |
| Gain formula | −R2 / R1 = −3 | 1 + R2/R1 = +3 |
| Output amplitude | 6 V p-p | 6 V p-p |
| Phase shift | 180° (inverted) | 0° (same phase) |
| CH A probe | Junction of VSINE and R1 | Junction of VSINE and pin 3 |
| CH B probe | Pin 6 (output) | Pin 6 (output) |

Circuit Analysis Lab 7  —  Proteus ISIS Simulation Procedure