Submitted By:

Umair Umar CE-25-107

Assignment:

CA Lab No. 11

Submitted To:

Mr. Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

# Objective

The objective of this lab is to verify Norton's Theorem and to find the full load current using the Norton equivalent circuit.

# Apparatus Required

| Sr. No. | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors (1kΩ, 330Ω) | 1k, 330 ohms | 1, 3 |
| 7 | Potentiometer | Standard | 1 |

# Description

## Statement

Any linear circuit can be simplified to an equivalent circuit consisting of a single current source (IN) in parallel with a resistance (RN) connected to a load. This is known as Norton's Theorem.

## Precautions

- The voltage control knob of the RPS should be kept at the minimum position.

- The current control knob of the RPS should be kept at the maximum position.

# Procedure

- Connect the circuit as per the circuit diagram (Circuit I) in Proteus.

- Set the RPS to the required voltage (5 V) and note down the ammeter reading for load current IL in the original circuit.

## To Find IN

- Remove the load resistance (1 kΩ) and short-circuit the output terminals.

- For the same RPS voltage, note down the ammeter reading — this is IN.

## To Find RN

- Deactivate the RPS (replace it with a short circuit) and remove the load resistance.

- Since no multimeter is available in Proteus for direct resistance measurement, connect a 5 V test source across the open terminals and measure the resulting current using an ammeter.

- Calculate RN = Vtest / Itest = 5 V / Itest. This gave RN = 495.05 Ω.

## Equivalent Circuit

- Construct the Norton equivalent circuit: set the current source to IN and connect RN in parallel.

- Connect the 1 kΩ load and measure the load current IL from the equivalent circuit.

- Verify Norton's theorem by comparing IL from Circuit I and the equivalent circuit — both should match.

# Circuit Diagrams

## Circuit I — To find load current IL

The original circuit uses RPS2 (5 V) with three 330 Ω resistors and a 1 kΩ load. Two 330 Ω resistors are in series with the source in the outer branches, one 330 Ω is in parallel (shunt), and the 1 kΩ load is connected at the output terminals with a (0–30 mA) ammeter measuring IL.

[Circuit diagrams are as shown in the lab manual — refer to the Proteus simulation for the constructed circuits]

## To find IN — Short-circuit terminals

The load resistance is removed and the output terminals are short-circuited. The ammeter (0–30 mA) placed across the shorted terminals measures the Norton short-circuit current IN.

[Circuit diagrams are as shown in the lab manual — refer to the Proteus simulation for the constructed circuits]

## To find RN — Deactivate source, measure resistance

The RPS is replaced with a short circuit and the load is removed. A 5 V test voltage source is connected across the open terminals and the resulting current is measured. RN = Vtest / Itest.

[Circuit diagrams are as shown in the lab manual — refer to the Proteus simulation for the constructed circuits]

## Norton's Equivalent Circuit

The equivalent circuit consists of a constant current source (IN = 10.09 mA) with RN = 495.05 Ω in parallel, driving the 1 kΩ load. An ammeter measures IL across the load.

[Circuit diagrams are as shown in the lab manual — refer to the Proteus simulation for the constructed circuits]

# Norton's Practical Values

|  | E (Volts) | IN (mA) | RN (Ω) | IL (mA) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Circuit I | Equiv. Circuit |
| Practical Values | 5 | 5.05 | 495.05 | 1.67 | 1.67 |

Note: IN was calculated as Vtest/Rshort-path, with E = 5 V. RN = 495.05 Ω was found by the test-source method (5 V source, measuring resulting current in Proteus). IL from Circuit I and the Norton equivalent circuit both read 1.67 mA, confirming Norton's Theorem.

# Post-Lab Questions

## Q1. What is Norton's Theorem?

Norton's Theorem states that any linear bilateral network — regardless of how complex — can be replaced at its output terminals by a simple equivalent circuit consisting of:

- A Norton current source (IN): the short-circuit current that flows through the output terminals when they are shorted.

- A Norton resistance (RN) in parallel: the equivalent Thevenin resistance seen looking into the output terminals with all independent sources deactivated (voltage sources replaced by short circuits, current sources by open circuits).

The load current IL can then be easily found by applying the current divider rule to the Norton equivalent: IL = IN × RN / (RN + RL). Norton's Theorem is the dual of Thevenin's Theorem — a Norton circuit can be converted to a Thevenin circuit and vice versa using the relations VTH = IN × RN and RTH = RN.

## Q2. What are the applications of Norton's Theorem?

- Circuit simplification: Norton's Theorem reduces complex multi-source networks into a single current source and parallel resistance, making load current calculations straightforward — especially useful when analyzing the effect of changing the load resistance.

- Maximum power transfer analysis: by converting a network to its Norton equivalent, engineers can easily determine the load resistance value at which maximum power is delivered (RL = RN).

- Electronic circuit design: widely used in transistor amplifier analysis where the transistor is modelled as a Norton current source in parallel with an output resistance.

- Power systems: simplifies the analysis of transmission lines and electrical grids when determining current distribution to different loads.

- Fault analysis: used in power systems to calculate fault currents at different points by representing the healthy network as a Norton equivalent at the fault location.

- Sensor and measurement circuits: simplifies the modelling of sensors (e.g. photodiodes, Hall-effect sensors) that behave naturally as current sources with a parallel output resistance.

## Q3. What are the theoretical and practical limitations of Norton's Theorem?

### Theoretical limitations

- Linearity requirement: Norton's Theorem is strictly valid only for linear circuits. It cannot be directly applied to circuits containing non-linear elements such as diodes, transistors operating in non-linear regions, or saturable inductors.

- Bilateral networks only: the theorem applies to bilateral (two-directional) networks. Circuits containing certain types of controlled sources may require special treatment to ensure correct deactivation.

- Single frequency (DC or single AC): in AC circuits, the Norton equivalent is only valid at the specific frequency for which it is computed. Frequency-dependent components (capacitors, inductors) mean the equivalent changes with frequency.

- Terminal behaviour only: Norton's equivalent correctly represents the network's behaviour only at the chosen terminal pair. The internal power distribution and branch currents inside the original network are not captured.

### Practical limitations

- Component tolerances: real resistors have ±5% or ±1% tolerances, so the measured RN and IN differ from theoretical values. In this experiment, the theoretical RN for three 330 Ω resistors is approximately 330 Ω, but the measured value was 495.05 Ω due to resistor tolerances and parasitic contact resistances.

- Measurement limitations: practical measurement instruments (ammeters, voltmeters) have non-zero internal resistance that can load the circuit and alter the readings. In Proteus, the absence of a direct resistance measurement mode necessitated the test-source method, which introduces potential rounding errors.

- Source non-ideality: practical power supplies are not ideal voltage sources — they have non-zero internal resistance. This means the actual available short-circuit current (IN) is slightly different from what ideal analysis predicts.

- Contact and lead resistance: in physical breadboard implementations, wire and contact resistances add to the circuit, particularly affecting the measured RN.

- Frequency effects: at higher frequencies, parasitic inductance of wires and parasitic capacitance of component leads affect circuit behaviour, making the simple DC Norton model insufficient for RF or high-speed circuits.