Submitted By:

Umair Umar CE-25-107

Assignment:

CA Lab No. 6

Submitted To:

Mr. Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

Proteus Procedure

Lab 6 Verification of Kirchhoff's Laws (KCL & KVL)

PART A — KCL Circuit Simulation

Circuit: RPS (Battery) → 330Ω in series → splits into two parallel branches: [1kΩ] and [220Ω + 1kΩ in series]. Three ammeters measure I1, I2, and I3.

A1. Opening Proteus and Starting a New Project

Step 1:  Open Proteus ISIS on your computer.

Step 2:  Click File → New Project. A wizard will appear.

Step 3:  Give your project a name (e.g., Lab6_KCL) and choose a save location.

Step 4:  Click Next through all wizard pages, then click Finish. A blank schematic sheet will open.

A2. Adding Components from the Device List

Step 5:  On the left toolbar, click the Component Mode button (it looks like a chip/IC icon). This activates the device picker.

Step 6:  Click the P button (Pick Device) at the top of the left panel. The Component Search window opens.

Step 7:  In the search box, type BATTERY and press Enter. Select BATTERY from the results list and click OK. It will appear in your devices list on the left.

Step 8:  Click P again. Type RES and press Enter. Select RES from the results and click OK.

Note:  You will need 4 resistors total (330Ω, 1kΩ, 220Ω, 1kΩ) you only need to add RES once, then place it 4 times.

A3. Adding Ammeters (Virtual Instruments)

Step 9:  On the left toolbar, click the Virtual Instruments Mode button (it looks like an oscilloscope icon).

Step 10:  In the instruments list that appears, find and click AMMETER.

Step 11:  Place 3 ammeters on the schematic by clicking on the sheet one for I1, one for I2, one for I3.

Note:  Ammeters are NOT found in the device list (P button). They are only in Virtual Instruments Mode.

A4. Placing Components on the Schematic

Step 12:  Click on BATTERY in your devices list (left panel). Click on the schematic to place it on the left side.

Step 13:  Click on RES in your devices list. Place 4 resistors on the schematic in the positions shown in the circuit diagram.

Tip:  Press R to rotate a component before placing it.

A5. Setting Component Values

Step 14:  Double-click the Battery. In the properties window, set Voltage to 5 (for the first reading). Click OK.

Step 15:  Double-click R1 (the 330Ω resistor). Set Resistance to 330. Click OK.

Step 16:  Double-click R2 (the 1kΩ in I2 branch). Set Resistance to 1k. Click OK.

Step 17:  Double-click R3 (the 220Ω in I3 branch). Set Resistance to 220. Click OK.

Step 18:  Double-click R4 (the 1kΩ in I3 branch). Set Resistance to 1k. Click OK.

A6. Wiring the KCL Circuit

Step 19:  Hover over the + terminal of the Battery. A green pencil cursor appears click to start a wire.

Step 20:  Draw the wire to the left terminal of R1 (330Ω). Click to complete the connection.

Step 21:  Connect the right terminal of R1 to the + terminal of Ammeter A1 (I1). Connect the − terminal of A1 to the node junction point.

Step 22:  From the junction node, draw a wire upward to the + terminal of Ammeter A3 (I3). Connect the − terminal of A3 to the left terminal of R3 (220Ω). Connect the right terminal of R3 to R4 (1kΩ). Connect the other end of R4 back to the − terminal of the Battery.

Step 23:  From the same junction node, draw a wire downward to the + terminal of Ammeter A2 (I2). Connect the − terminal of A2 to R2 (1kΩ). Connect the other end of R2 back to the − terminal of the Battery.

Note:  Make sure both parallel branches connect back to the negative terminal of the battery — this completes the circuit loop.

A7. Running the KCL Simulation

Step 24:  Click the green Play button at the bottom left of the Proteus window to start simulation.

Step 25:  Read the values shown on Ammeter A1 (I1), A2 (I2), and A3 (I3). Record them in the Practical Values table.

Step 26:  Verify that I1 = I2 + I3 to confirm KCL.

Step 27:  Click the Stop button (red square). Double-click the Battery and change the voltage to 15V. Run again and record readings.

Step 28:  Repeat Step 27 for 25V.

Expected Theoretical Values for KCL:

| Sl. No. | Voltage E (V) | I1 (mA) | I2 (mA) | I3 (mA) | I1=I2+I3 (mA) |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 | 5.68 | 3.12 | 2.56 | 5.68 |
| 2 | 15 | 17.06 | 9.37 | 7.69 | 17.06 |
| 3 | 25 | 28.43 | 15.62 | 12.81 | 28.43 |

PART B — KVL Circuit Simulation

Circuit: RPS (Battery) → 330Ω (V1) → splits into two parallel branches: [1kΩ (V2)] and [220Ω (V3)]. Three voltmeters measure V1, V2, and V3.

B1. Starting a New Schematic for KVL

Step 1:  It is recommended to save your KCL file first, then open a new project (File → New Project) named Lab6_KVL.

Step 2:  Alternatively, clear the schematic (Edit → Select All → Delete) and reuse the same project.

B2. Adding Components

Step 3:  Click the P button. Search and add BATTERY (already in list if reusing project).

Step 4:  Click P again. Search and add RES. You need 3 resistors: 330Ω, 1kΩ, 220Ω.

B3. Adding Voltmeters (Virtual Instruments)

Step 5:  Click Virtual Instruments Mode on the left toolbar.

Step 6:  Select VOLTMETER from the list. Place 3 voltmeters on the schematic — one for V1, one for V2, one for V3.

Note:  Voltmeters are also only in Virtual Instruments Mode, not in the device list.

B4. Setting Component Values

Step 7:  Double-click the Battery. Set Voltage to 5. Click OK.

Step 8:  Double-click R1. Set Resistance to 330. Click OK.

Step 9:  Double-click R2. Set Resistance to 1k. Click OK.

Step 10:  Double-click R3. Set Resistance to 220. Click OK.

B5. Wiring the KVL Circuit

Step 11:  Connect the + terminal of Battery to the left terminal of R1 (330Ω).

Step 12:  Connect the right terminal of R1 to a node junction point (this is where the circuit splits).

Step 13:  From the junction, draw one wire to R2 (1kΩ). Connect the other end of R2 to the − terminal of the Battery.

Step 14:  From the same junction, draw another wire to R3 (220Ω). Connect the other end of R3 to the − terminal of the Battery.

Step 15:  Place Voltmeter V1 in parallel across R1  connect its + terminal to the left side of R1 and its − terminal to the right side of R1.

Step 16:  Place Voltmeter V2 in parallel across R2  connect its + and − terminals across R2.

Step 17:  Place Voltmeter V3 in parallel across R3  connect its + and − terminals across R3.

Note:  V2 and V3 will show equal readings because R2 and R3 are in parallel — they share the same two nodes.

B6. Running the KVL Simulation

Step 18:  Click the Play button to start simulation.

Step 19:  Read V1, V2, and V3 from the three voltmeters. Record in the Practical Values table.

Step 20:  Verify that E1 = V1 + V2 (not V1+V2+V3 since V2 and V3 are the same parallel voltage).

Step 21:  Stop simulation. Change Battery to 10V. Run again and record readings.

Step 22:  Repeat for 15V.

Expected Theoretical Values for KVL:

| Sl. No. | RPS E1 (V) | V1 (V) | V2 (V) | V3 (V) | E1=V1+V2 (V) |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 | 3.23 | 1.77 | 1.77 | 5.00 |
| 2 | 10 | 6.47 | 3.53 | 3.53 | 10.00 |
| 3 | 15 | 9.70 | 5.30 | 5.30 | 15.00 |

Common Mistakes to Avoid

| Mistake | Correct Approach |
| --- | --- |
| Ammeter placed in parallel | Always place Ammeter in SERIES with the branch |
| Voltmeter placed in series | Always place Voltmeter in PARALLEL across the component |
| Searching for Ammeter/Voltmeter in device list (P button) | Find them in Virtual Instruments Mode on the left toolbar |
| Circuit not forming a complete loop | Both parallel branches must connect back to the Battery negative terminal |
| Adding V2 + V3 separately in KVL | V2 and V3 are the same parallel voltage — only add V1 + V2 for KVL check |
| Getting negative ammeter reading | Reverse the ammeter direction — current must enter the + terminal |