Circuit Analysis — Lab 3

Proteus Simulation Procedure

Task 1: DC Series Circuit (5V, R1 = 5.1kΩ, R2 = 3.3kΩ)

Step 1: Place the AC Source

- Clicked the P button in the left panel to open the Pick Device window.

- Searched for BATTERY in the search bar and placed it on the schematic.

- Double clicked the battery and set the voltage to 5V.

Step 2: Place the Resistors

- Searched for RES in the Pick Device window and placed two resistors on the schematic.

- Double clicked R1 and set its value to 5.1k.

- Double clicked R2 and set its value to 3.3k.

Step 3: Add Ground

- Added a GND terminal from the terminals menu on the left toolbar and connected it to the negative terminal of the source.

Step 4: Connect the Circuit

- Connected wires to form a single series loop: Source(+) → R1 → R2 → GND.

Step 5: Place Voltmeters

- Clicked the Virtual Instruments icon on the left toolbar and selected VOLTMETER.

- Placed first voltmeter across R1 with (+) probe on the wire before R1 and (−) probe on the wire after R1.

- Placed second voltmeter across R2 with (+) probe before R2 and (−) probe after R2.

- Double clicked each voltmeter and set mode to DC Volts.

Step 6: Place Ammeter

- Selected AMMETER from the Virtual Instruments menu.

- Deleted the wire between the source and R1 to create a gap.

- Inserted the ammeter in that gap in series with the circuit.

- Double clicked the ammeter and set mode to DC Milliamps.

Step 7: Run Simulation

- Clicked the Play button (▶) at the bottom left to start the simulation.

- Recorded the voltmeter and ammeter readings from the display.

Task 2: DC Series Circuit (9V, R1 = 3kΩ, R2 = 10kΩ, R3 = 5kΩ)

Step 1: Place the DC Source

- Clicked the P button and searched for BATTERY in the Pick Device window.

- Placed the battery on the schematic and double clicked it to set the voltage to 9V.

Step 2: Place the Resistors

- Searched for RES and placed three resistors on the schematic.

- Set R1 = 3k, R2 = 10k, R3 = 5k by double clicking each resistor.

Step 3: Add Ground

- Added a GND terminal and connected it to the negative terminal of the battery.

Step 4: Connect the Circuit

- Connected wires to form a single series loop: Battery(+) → R1 → R2 → R3 → GND.

Step 5: Place Voltmeters

- Placed three voltmeters across R1, R2 and R3 respectively in parallel.

- Set all voltmeters to DC Volts mode.

Step 6: Place Ammeters at Node 2 and Node 3

- Deleted the wire between R1 and R2 to create a gap at Node 2.

- Inserted first ammeter in that gap to measure current at Node 2.

- Deleted the wire between R2 and R3 to create a gap at Node 3.

- Inserted second ammeter in that gap to measure current at Node 3.

- Set both ammeters to DC Milliamps mode.

Step 7: Run Simulation

- Clicked the Play button (▶) to start the simulation.

- Recorded all voltmeter and ammeter readings.

- Observed that current at Node 2 and Node 3 were equal (0.5 mA) confirming the series circuit rule.