Submitted By:

Umair Umar CE-25-107

Assignment:

CA Lab No. 1

Submitted To:

Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

- Define Oscilloscope and write its applications.

An oscilloscope is an electronic test instrument that visually displays electrical signals as waveforms on a screen, showing how voltage changes over time. It plots voltage on the vertical axis and time on the horizontal axis.

Applications:

- Measuring voltage, frequency, and period of electrical signals

- Analyzing waveform shapes (sine, square, triangle)

- Debugging and troubleshooting electronic circuits

- Measuring phase difference between two signals

- Checking the output of a signal generator

- Analyzing audio and communication signals

- What is Peak to Peak Voltages?

Peak-to-peak voltage (V(_{pp})) is the total voltage difference between the maximum positive peak and the maximum negative peak of a waveform. It represents the full swing of the signal. For a symmetric waveform, it is equal to twice the peak voltage. For example, if a sine wave reaches +4V at its highest point and −4V at its lowest, the peak-to-peak voltage is 8V. It is measured on an oscilloscope by observing the total vertical height of the waveform.

- What is RMS value?

The RMS (Root Mean Square) value of an AC signal is the equivalent DC voltage that would produce the same amount of power in a resistive load. It is calculated by squaring the instantaneous values of the waveform, taking their average, and then finding the square root of that average. The RMS value is the most practical way to express AC voltage because it directly relates to power delivery. For example, household supply of 220V AC refers to its RMS value, not its peak value.

- Write the RMS values of Sine, Square and Triangular wave?

The RMS values for different waveforms with peak voltage V(_{p}) are:

- Sine Wave: V({rms}) = V({p}) / √2 ≈ 0.707 × V(_{p})

- Square Wave: V({rms}) = V({p}) (RMS equals peak value because the wave is always at maximum)

- Triangular Wave: V({rms}) = V({p}) / √3 ≈ 0.577 × V(_{p})

The square wave has the highest RMS value relative to its peak, meaning it delivers the most power compared to sine and triangular waves of the same peak voltage.

5.	Explain DC offset?

DC offset refers to the addition of a constant DC voltage to an AC signal, which shifts the entire waveform up or down from the zero reference line. For example, a sine wave normally oscillates symmetrically around 0V. If a DC offset of +3V is applied, the entire wave shifts upward so it now oscillates between −1V and +7V instead of −4V and +4V, while its shape and frequency remain unchanged. DC offset is important in circuit design because it can affect how components like capacitors and transistors respond to a signal, and it is adjustable on a signal generator using the Offset control.