Submitted By:

Umair Umar CE-25-107

Assignment:

CA Lab No. 5

Submitted To:

Mr. Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

Circuit Analysis Lab 5

Procedure Document: Proteus

| Experiment | Lab 5 |
| --- | --- |
| Software | Proteus |
| Task 1 Source | VSINE (AC sine wave, 5 V peak) |
| Task 2 Source | BATTERY (DC, 9 V) |
| Objective | Simulate resistive circuits and measure voltages and currents |

Part 1 — General Proteus Setup (Common to Both Tasks)

The following steps apply to every simulation in Proteus ISIS and must be completed before building any specific circuit.

1.1  Opening a New Design

1.   Launch Proteus ISIS Professional on your computer.

2.   Go to File → New Design. The Create New Design dialogue will appear.

3.   Select the DEFAULT template and click OK. A blank schematic editing window will open.

1.2  Adding Components from the Library

1.   Press the P key on the keyboard, OR click the letter P above the Object Selector on the left panel. The Pick Devices (Library Browser) window opens.

2.   Type the component name in the Keywords field. Matching results appear in the centre list.

3.   Double-click the correct result to add it to the Object Selector. Repeat for every component needed.

4.   Close the Library Browser once all components have been selected.

1.3  Placing Components on the Schematic

1.   Left-click a component name in the Object Selector to highlight it. A preview appears in the Overview Window (top-right corner).

2.   Left-click anywhere on the Editing Window to drop the component at that position.

3.   To rotate before placing, press R. To mirror, press M.

4.   Repeat for all components, spacing them to match the target circuit layout.

1.4  Editing Component Properties

1.   Right-click any placed component and select Edit Properties (or double-click it).

2.   Change the relevant fields — for example, resistance value for a resistor, or voltage/frequency for a source.

3.   Click OK to confirm. The label on the schematic will update automatically.

1.5  Adding Ground and Power Terminals

1.   Right-click an empty area of the Editing Window → Place → Terminal.

2.   Select GROUND to place a 0 V reference terminal. Connect it to the negative/return node of your circuit.

3.   If a fixed DC rail is needed, select POWER and set its value (e.g. 5 V).

1.6  Wiring Components

1.   Hover over a component pin until the cursor changes to a green crosshair.

2.   Left-click and drag to the destination pin, then release to complete the wire. Proteus auto-routes right-angle bends.

3.   A junction dot appears automatically wherever three or more wires meet at the same point.

4.   To delete a wire segment, right-click it and select Delete Wire.

1.7  Adding Virtual Instruments (Voltmeter / Ammeter)

1.   Right-click the Editing Window → Place → Virtual Instrument.

2.   Select DC VOLTMETER to measure voltage. Connect its two probe terminals across (in parallel with) the component you want to measure.

3.   Select DC AMMETER to measure current. Break the wire at the point of measurement and insert the ammeter in series so that current flows through it.

Note: Voltmeters go in parallel. Ammeters go in series. Swapping these will give wrong readings or short-circuit the source.

1.8  Running and Stopping the Simulation

1.   Click the green Play button (▶) in the Animation Panel at the bottom-left to start.

2.   Instrument displays will update in real time. Read the voltmeter and ammeter values now.

3.   Click the Stop button (■) to end the simulation before making any circuit changes.

Part 2 — Task 1: Series AC Circuit (VSINE + R1 + R2)

Circuit: VSINE AC source (5 V peak) with R1 = 5.1 kΩ and R2 = 3.3 kΩ connected in series.

Measurements required: Voltage across R1, voltage across R2, and current between R1 and R2.

Step 1 — Add the AC Source (VSINE)

1.   Open the Library Browser (press P).

2.   Type VSINE in the Keywords field. Double-click VSINE from the results to add it to the Object Selector.

3.   Place VSINE on the left side of the schematic.

4.   Right-click VSINE → Edit Properties. Set the following:

Amplitude (peak voltage): 5 V

Frequency: 50 Hz

5.   Click OK.

Note: VSINE produces a sine wave with the set peak value. Proteus multimeters display RMS voltage, not peak voltage. The RMS value of a 5 V peak sine is 5 / √2 ≈ 3.54 V. This is why measured values are lower than expected from peak arithmetic.

Step 2 — Add and Configure Resistors

1.   Open the Library Browser. Type RES and double-click to add a resistor. Add it twice (for R1 and R2).

2.   Place both resistors to the right of VSINE.

3.   Right-click R1 → Edit Properties → set resistance to 5.1k. Click OK.

4.   Right-click R2 → Edit Properties → set resistance to 3.3k. Click OK.

Step 3 — Add Measurement Instruments

1.   Right-click the Editing Window → Place → Virtual Instrument → DC VOLTMETER. Place one voltmeter and connect its probes across R1 (in parallel).

2.   Repeat to place a second DC VOLTMETER across R2.

3.   Right-click → Place → Virtual Instrument → DC AMMETER. Place it between R1 and R2 — break the series wire at that junction and insert the ammeter inline so all current passes through it.

Step 4 — Wire the Series Circuit

1.   Connect the positive terminal of VSINE to the top pin of R1.

2.   Connect the bottom pin of R1 → ammeter input → ammeter output → top pin of R2.

3.   Connect the bottom pin of R2 back to the negative terminal of VSINE.

4.   Place a GROUND terminal at the negative terminal of VSINE.

Note: In a series circuit the same current flows through every component. There is only one path for current, so one ammeter placed anywhere in the loop gives the current for the whole circuit.

Step 5 — Simulate and Record

1.   Click Play (▶) to start the simulation.

2.   Read and record the voltmeter display for V_R1 and V_R2.

3.   Read and record the ammeter display for the series current I.

4.   Click Stop (■) when done.

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 2.14 V | 1.39 V | 0.42 mA |

Part 3 — Task 2: Parallel DC Circuit (BATTERY + R1 + R2 + R3)

Circuit: 9 V BATTERY with R1 = 10 kΩ, R2 = 2 kΩ, and R3 = 1 kΩ all connected in parallel between the same two nodes.

Measurements required: Current through each resistor individually, and voltage between node 2 and node 3.

Step 1 — Add the DC Source (BATTERY)

1.   Open the Library Browser (press P).

2.   Type BATTERY in the Keywords field. Double-click the result to add it.

3.   Place the BATTERY on the left side of the schematic.

4.   Right-click BATTERY → Edit Properties → set Voltage to 9 V. Click OK.

Note: BATTERY in Proteus provides a constant DC voltage with no AC component. The full 9 V appears directly across every branch of a parallel circuit, so no RMS conversion is needed.

Step 2 — Add and Configure Three Resistors

1.   Open the Library Browser. Type RES and add three resistor instances to the Object Selector.

2.   Place R1, R2, and R3 vertically side by side to the right of the battery, with space between them.

3.   Right-click each and set their values: R1 = 10k,  R2 = 2k,  R3 = 1k. Click OK for each.

Step 3 — Wire the Parallel Circuit

1.   Draw a horizontal top rail wire from the positive terminal of the battery across to the top pin of R1, then continue to R2, then R3. All top pins connect to this same rail (this is nodes 1–2–3–4 in the circuit diagram).

2.   Draw a horizontal bottom rail wire from the negative terminal of the battery across to the bottom pin of R1, then R2, then R3 (nodes 8–7–6–5).

3.   Place a GROUND terminal at the negative terminal of the battery (node 8).

Note: Nodes 2 and 3 are both points on the same top rail wire. Because they are directly connected by a wire with no component between them, the voltage difference between node 2 and node 3 is 0 V.

Step 4 — Add Three Ammeters and One Voltmeter

1.   For each resistor branch, break the top wire above that resistor and insert a DC AMMETER in series so that all current entering that branch passes through the ammeter.

2.   Place Ammeter 1 in the branch above R1.

3.   Place Ammeter 2 in the branch above R2.

4.   Place Ammeter 3 in the branch above R3.

5.   Place a DC VOLTMETER with one probe at node 2 and the other at node 3 on the top rail.

Note: Each parallel branch has its own independent current, so a separate ammeter is required per branch. You cannot measure all three currents with a single ammeter here (unlike the series circuit in Task 1).

Step 5 — Simulate and Record

1.   Click Play (▶) to start the simulation.

2.   Read and record the ammeter display for I_R1, I_R2, and I_R3 from their respective ammeters.

3.   Read and record the voltmeter display for the voltage between node 2 and node 3.

4.   Click Stop (■) when done.

| R1 Current | R2 Current | R3 Current | Voltage (Node 2–3) |
| --- | --- | --- | --- |
| 0.9 mA | 4.5 mA | 9.0 mA | 0 V |